Radius 10 Corners

When you see corners with a radius, the distance from the edges to the centre of the corner is equal to the radius.

In this case, R10, the distance from each edge to the centre of the corner is 10mm.

There are many ways in which you can mark these out.

_____________________________________________________________

The Big Hole

Method 1

Drill a number of Ø7 holes inside the big circle.  Use a Junior Hacksaw to cut between the holes.  Use a half-round file to complete the hole.

This method is quicker but there is a lot of filing.

Method 2

Drill a small hole, (about Ø3), and use a Piercing Saw to cut close to the line.  Use a half-round file to finish the hole.

This method requires less filing but the blades of a Piercing Saw are very delicate and are easy to break.  Patience is required with this method.