Pac-Man How To Make

1.  	Scribe an X in the square to find the centre of the piece.

2. 	Punch the centre of the X.

3.	Set your Dividers to 20mm and scribe a circle with the punch mark as centre.

4.	Measure 15mm from the centre of the circle along one of the diagonals and mark.

5.	Punch where the mark crosses the diagonal.

6.	Drill a Ø4mm hole at this latest punch mark.

7.	Hacksaw off the corners of the piece.

8.	Hacksaw out the ‘mouth’ of the piece.

9.	File the piece to shape.